# LCD Inquiry Specs

## Dimensions

*View area： W 42.65 x H 18.15 mm				*Glass thickness：

Active area：W          x H          mm 				1.1 mm front

Front glass：W 50.80 x H 30.45 mm				1.1 mm rear

Rear glass：W 50.80 x H 23.00 mm

Critical dimensions:

- Front glass Width (Fw)

- Top of front glass(connector) - centered Zebra width(Zw) with equal edges on both sides

Other dimensions are not that important but still prefered to be as specified.

## View Area Layout:

## LCD Connection:

- Type: Elastomeric Zebra connector (already fabricated)

- 40 terminals (schema below)

- 38 segment

- 2 common (39 & 40)

- Terminal width: 0.6mm

- Gap between terminals: 0.4mm

- 40 terminals + 39 gaps between them: 40*0.6 + 39*0.4 = 39.6mm (Zw from first drawing)

## Polarizer:

- front: anti-UV, attached

- rear: attached

- reflective type

## Display characteristics:

- Twisted Nematic (TN)

- Segment type

- Positive

## View Angle

## Temperature:

-10C ~ +50C

## LCD Driver:

OKI Semiconductor MSM5219B

Datasheet attached